Dual-Track MPC Planner - User Guide

Purpose of this document

This document explains the structure, usage and tuning possibilities of the dualtrack_mpc_planner.py module. It is written as a practical guide for a reader who wants to understand what the code does, how it is called from a simulation, which model parameters can be modified, and how these parameters influence the vehicle motion, path tracking, obstacle avoidance and numerical behaviour of the MPC solver.

Role of the module

The module implements a robust lateral MPC planner based on a dual-track two-degrees-of-freedom vehicle model. The planner receives the current vehicle state, a global reference path, the current longitudinal speed and optional circular obstacles. It extracts a local reference window, modifies this local reference around obstacles, builds a quadratic programming problem, solves it with OSQP, and returns the first steering command together with the predicted trajectory.

1. Basic Usage

The main external interface is the RobustDualTrackMPCPlanner class. A typical workflow consists of creating the parameter objects, building a reference path, calling plan() at every control step, applying the first steering command, and then updating the vehicle state.

from dualtrack_mpc_planner import VehicleParams, MPCParams, RobustDualTrackMPCPlanner, ReferencePath, Obstacle

vp = VehicleParams()
mp = MPCParams()
planner = RobustDualTrackMPCPlanner(vp, mp)

# Reference path from X-Y points
ref = ReferencePath.from_xy(X_points, Y_points)

# State vector: [vy, phi, r, X, Y]
state = np.zeros(5)
vx = mp.vx_ref
delta_prev = 0.0
obstacles = [Obstacle(X=30.0, Y=0.0, radius=0.9)]

result = planner.plan(state, ref, vx, delta_prev, obstacles)
delta_cmd = result.delta_apply
state_next = planner.step_vehicle(state, delta_cmd, vx)

2. Inputs and Outputs

| Item | Code representation | Meaning |
| --- | --- | --- |
| Vehicle state | state = [vy, phi, r, X, Y] | Lateral velocity, heading angle, yaw rate, global X position and global Y position. |
| Control input | delta_f | Front steering angle. The MPC optimizes a sequence of steering angles but only the first one is applied. |
| Longitudinal speed | vx | External scheduling parameter. It is not part of the state vector in this implementation. |
| Reference path | ReferencePath(X, Y, phi) | Global route to be followed. The planner extracts a local horizon from it at each step. |
| Obstacles | List[Obstacle] | Optional list of circular obstacles defined by centre position and radius. |
| Planner output | MPCResult | Contains solver status, predicted states, steering sequence, applied steering command, local reference and active obstacle corridor constraints. |

3. Code Architecture

| Block | Main responsibility |
| --- | --- |
| VehicleParams | Stores physical and geometric vehicle parameters used by the tire model, load transfer equations, yaw dynamics and obstacle safety offset. |
| MPCParams | Stores MPC tuning parameters: sample time, horizon length, tracking weights, stabilising weights, steering weights, constraints, obstacle parameters and OSQP settings. |
| ReferencePath | Stores the route coordinates and heading values. The from_xy() constructor automatically computes heading from X-Y points. |
| DualTrack2DOFModel | Implements the nonlinear vehicle model, the low-speed kinematic model, model blending, Euler discretisation and numerical linearisation. |
| RobustDualTrackMPCPlanner | Coordinates the complete MPC step: local reference extraction, obstacle-aware reference shaping, QP construction, OSQP solving, warm start handling and fallback steering. |
| MPCResult | Packages the result of one MPC call, including the first steering command and the predicted trajectory. |

4. Vehicle Parameters

VehicleParams describes the physical vehicle that is used by the prediction model. These parameters should be changed when modelling a different car, changing the geometric safety envelope or identifying a different tire behaviour.

| Parameter | Default | Meaning | Tuning effect |
| --- | --- | --- | --- |
| m | 1500.0 kg | Total vehicle mass. | Higher mass reduces acceleration response for the same lateral tire forces. |
| ms | 1320.0 kg | Sprung mass used in lateral load transfer. | Higher value increases left-right load transfer during cornering. |
| lf | 1.15 m | Distance from centre of gravity to front axle. | Changes yaw moment arm and static front axle load. |
| lr | 1.586 m | Distance from centre of gravity to rear axle. | Changes yaw moment arm and static rear axle load. |
| h | 0.43 m | Height-like distance between CG and roll axis. | Higher value increases load transfer and can reduce inner-wheel vertical load. |
| d | 0.77 m | Half track width. | Larger value reduces lateral load transfer for the same lateral acceleration. |
| Iz | 2400 kg m2 | Yaw moment of inertia. | Higher value makes yaw acceleration slower and the vehicle less responsive. |
| g | 9.81 m/s2 | Gravity. | Usually should not be changed. |
| C1, C2 | 13.098, -0.001045 | Empirical tire-force coefficients. | They define the lateral tire force sensitivity to slip angle and vertical load. |
| length | 4.698 m | Vehicle length for visualisation and safety envelope. | Mainly affects drawings and collision interpretation. |
| width | 1.829 m | Vehicle width for visualisation and obstacle safety offset. | Higher width increases required obstacle clearance. |

5. MPC Parameters and Tuning

5.1 Time Horizon and Speed

| Parameter | Default | Meaning | If increased | If decreased |
| --- | --- | --- | --- | --- |
| dt | 0.10 s | Sampling time of one MPC step. | Longer prediction time for same N, but coarser control resolution. | Finer control resolution, but more simulation and solver calls. |
| N | 50 | Number of prediction steps. | Longer look-ahead and better anticipation, but larger QP. | Faster QP, but shorter anticipation. |
| vx_ref_kmh | 50.0 km/h | Default longitudinal speed used by demos. | Higher speed increases travelled distance per horizon and makes steering constraints more critical. | Lower speed makes the route easier dynamically but may require more permissive steering rate at creep speeds. |
| vx_ref | vx_ref_kmh / 3.6 | Converted speed in m/s. | Derived value, not directly set. | Derived value, not directly set. |

5.2 Tracking Weights

| Parameter | Default | What it penalises | Practical tuning effect |
| --- | --- | --- | --- |
| QX | 35.0 | Longitudinal/global X position error. | Higher value keeps the predicted X positions closer to the reference but may reduce flexibility. |
| QY | 45.0 | Lateral/global Y position error. | Higher value improves lateral path tracking. Too high values can fight obstacle avoidance. |
| Qphi | 12.0 | Heading error. | Higher value aligns the vehicle direction more strongly with the reference path. |
| QX_terminal | 80.0 | Terminal X error. | Forces the end of the horizon closer to the reference. |
| QY_terminal | 110.0 | Terminal Y error. | Improves end-of-horizon lateral convergence. |
| Qphi_terminal | 30.0 | Terminal heading error. | Improves end-of-horizon orientation convergence. |

5.3 Stability and Comfort Weights

| Parameter | Default | What it penalises | Effect |
| --- | --- | --- | --- |
| Qvy | 3.0 | Lateral velocity vy. | Higher value reduces side-slip-like lateral motion and improves stability. |
| Qr | 8.0 | Yaw rate r. | Higher value reduces rapid yaw motion. |
| Qay_soft | 1.40 | Speed-dependent yaw/lateral acceleration behaviour through qr_eff = Qr + Qay_soft * vx^2. | Higher value makes the vehicle more conservative at higher speed. |

5.4 Steering Weights

| Parameter | Default | What it penalises | Effect |
| --- | --- | --- | --- |
| R_delta | 3.0 | Absolute steering angle delta_f. | Higher value produces smaller steering commands, but may worsen tracking in sharp curves. |
| R_d_delta | 100.0 | Steering increment between consecutive samples. | Higher value gives smoother steering and less oscillation, but slower reaction to curves or obstacles. |
| speed_adaptive_steering_weights() | function | Increases steering and steering-rate penalties with speed. | At higher speeds, the same steering action becomes more expensive, making the vehicle behaviour smoother and safer. |

6. Constraints

| Parameter / function | Default | Constraint role | Tuning recommendation |
| --- | --- | --- | --- |
| delta_max | 18 deg | Maximum front steering angle. | Use lower values for realistic road-speed steering; increase only if the vehicle cannot follow tight low-speed manoeuvres. |
| steering_rate_limit(vx) | speed dependent | Maximum steering change per MPC sample. | This is one of the most important comfort and stability limits. Increase for more agility, decrease for smoother behaviour. |
| phi_min / phi_max | -pi / +pi | Allowed heading range. | Usually keep at full angle range. |
| Y_min / Y_max | -300 / +300 m | Allowed global Y range. | Set according to road boundaries or simulation area. |
| ay_max | 6.0 m/s2 | Hard lateral acceleration approximation ¦vx*r¦ <= ay_max. | Lower values give safer, smoother behaviour; too low values can make sharp curves infeasible. |

The lateral acceleration constraint uses the approximation vx * r. It is simple and QP-friendly, but it should be interpreted as a planning constraint rather than a full tire-friction envelope.

7. Reference Path Handling

The planner does not optimize against the full route. Instead, extract_reference_window() cuts out a local horizon of N+1 points around the current vehicle position. The extraction is based on arclength projection, not only on the nearest array index. This is important on curved routes because the local reference remains aligned with the distance that the vehicle can actually travel over the horizon.

| Parameter | Default | Meaning |
| --- | --- | --- |
| reference_ds | None | If set, this fixed arclength spacing is used between local reference points. |
| reference_ds_min | 0.05 m | Minimum spacing used to avoid duplicate reference points at very low speeds. |
| Automatic ds | abs(vx) * dt | When reference_ds is None, the local path spacing follows the distance travelled per sample. |

8. Obstacle Avoidance

Obstacles are represented as circles. The code handles them in two complementary ways. First, it shifts the local reference path smoothly around the obstacle. Second, it creates local corridor constraints with slack variables, so the optimizer is encouraged to keep the predicted trajectory outside the safety boundary while still being able to return a feasible solution in difficult cases.

| Parameter | Default | Meaning | Effect of increasing |
| --- | --- | --- | --- |
| obstacle_clearance | 0.75 m | Extra safety distance around the obstacle and vehicle half-width. | Larger clearance creates a wider avoidance manoeuvre. |
| obstacle_reference_margin_min / max | 38 / 130 m | Minimum and maximum longitudinal range over which the reference is shifted. | The obstacle affects the local reference earlier and over a longer distance. |
| obstacle_reference_margin_time | 6.0 | Speed-dependent reference influence: time * ¦vx¦ clipped by min/max. | At the same speed, the avoidance begins earlier. |
| obstacle_constraint_margin_min / max | 6 / 18 m | Longitudinal range where corridor constraints are activated. | Harder safety constraints become active farther from the obstacle. |
| obstacle_constraint_margin_time | 0.8 | Speed-dependent corridor range. | The hard corridor zone grows with speed. |
| obstacle_soft_extra_margin | 0.40 m | Extra margin for the shifted reference beyond the hard safety offset. | The local reference is placed farther away from the obstacle. |
| obstacle_lateral_activation_margin | 2.7 m | Fade-out width for obstacle influence in lateral direction. | The obstacle affects a wider lateral region. |
| obstacle_reference_weight | 0.90 | Strength of reference displacement. | The local reference is pulled more strongly around the obstacle. |
| obstacle_preferred_side | +1 | Avoidance side: +1 local-left, -1 local-right, 0 automatic. | Positive/negative value fixes the side; zero lets the code choose automatically. |
| obstacle_slack_weight | 2000 | Penalty for violating corridor constraints. | Higher value makes obstacle corridor violation more expensive. |

The hard safety offset is computed as: obstacle radius + half vehicle width + obstacle_clearance. The soft reference offset adds obstacle_soft_extra_margin to this value. Therefore, changing the vehicle width or obstacle clearance directly changes the minimum obstacle distance used by the planner.

9. Vehicle Model Implementation

9.1 State and Input Definition

The model has five states and one control input:

| Symbol / index | Meaning |
| --- | --- |
| x[0] = vy | Lateral velocity in the vehicle frame. |
| x[1] = phi | Vehicle heading angle. |
| x[2] = r | Yaw rate. |
| x[3] = X | Global X position. |
| x[4] = Y | Global Y position. |
| u = delta_f | Front steering angle. |

9.2 Dynamic and Kinematic Model Blending

At normal road speeds, the prediction model is mainly the dual-track tire-force model. At very low speeds, the classical slip-angle formulas become poorly conditioned because they contain terms divided by longitudinal speed. To avoid this, the code blends a kinematic bicycle-type model with the dynamic model. Below about 2 m/s the kinematic model dominates; above about 6 m/s the dynamic model dominates; between them smoothstep() provides a continuous transition.

9.3 Tire Forces and Load Transfer

The dynamic model computes slip angles, estimates lateral acceleration as vx*r, calculates vertical loads on the four wheels, and then computes lateral tire forces from an empirical quadratic tire model. The vertical load model includes left-right load transfer caused by lateral acceleration, so the two wheels on the same axle may have different normal loads during cornering.

10. QP Construction and Solver Behaviour

The function build_qp() constructs the quadratic program solved by OSQP. The decision variables are all predicted states X_0 ... X_N, all steering commands U_0 ... U_{N-1}, and one slack variable for each active obstacle corridor constraint.

| QP component | Implementation in the code |
| --- | --- |
| Objective function | Reference tracking terms for X, Y and phi; stabilising terms for vy and r; steering magnitude penalty; steering-rate penalty; obstacle slack penalty. |
| Equality constraints | Initial state equality and linearised discrete-time dynamics. |
| Inequality constraints | Heading bounds, Y bounds, approximate lateral acceleration bound, steering angle bound, steering-rate bound, non-negative slack variables and obstacle corridor half-space constraints. |
| Linearisation | DualTrack2DOFModel.linearize_discrete() numerically computes A, B and c around the current trajectory guess. |
| Warm start | The previous predicted trajectory and steering sequence are shifted by one sample and reused as the next initial guess. |

10.1 Solver Parameters

| Parameter | Default | Meaning | When to modify |
| --- | --- | --- | --- |
| osqp_verbose | False | Print OSQP solver information. | Enable for debugging solver convergence. |
| osqp_polish | False | Enable OSQP polishing. | Can improve solution accuracy but may increase runtime. |
| osqp_max_iter | 2500 | Maximum OSQP iterations. | Increase if the solver often reaches the iteration limit. |
| osqp_eps_abs | 1e-3 | Absolute solver tolerance. | Decrease for more accuracy, increase for speed. |
| osqp_eps_rel | 1e-3 | Relative solver tolerance. | Decrease for more accuracy, increase for speed. |
| sqp_iterations | 1 | Number of sequential QP iterations per control step. | Increase if one linearisation is not accurate enough, but runtime will increase. |
| sqp_alpha | 1.0 | Damping factor for SQP update. | Reduce below 1.0 if repeated SQP iterations become unstable. |

If the solver fails, solve_mpc() clears the warm start and applies a safe fallback: the steering command is gradually moved back toward zero according to the speed-adaptive steering-rate limit.

11. Practical Tuning Guide

| Observed behaviour | Parameters to check first | Typical action |
| --- | --- | --- |
| Vehicle cuts corners or leaves the path. | QY, Qphi, terminal weights, N, reference_ds. | Increase QY/Qphi or terminal weights; increase horizon if anticipation is too short. |
| Vehicle oscillates around the reference path. | R_d_delta, Qr, Qvy, steering_rate_limit(). | Increase steering-rate penalty or yaw/lateral-velocity penalties; reduce steering-rate limit. |
| Vehicle cannot avoid obstacles in time. | obstacle_reference_margin_time, obstacle_reference_weight, delta_max, steering_rate_limit(). | Start avoidance earlier, increase reference displacement, or allow more steering authority. |
| Obstacle avoidance is too aggressive. | obstacle_reference_weight, obstacle_clearance, R_d_delta. | Reduce reference weight or clearance; increase steering-rate penalty for smoother manoeuvres. |
| QP is infeasible or often fails. | Y bounds, ay_max, delta_max, steering_rate_limit(), obstacle_slack_weight. | Relax hard limits or reduce obstacle aggressiveness; check whether the reference path is physically possible. |
| Computation time is too high. | N, osqp tolerances, sqp_iterations. | Reduce horizon, loosen tolerances, keep sqp_iterations at 1 for real-time tests. |
| Low-speed behaviour is unstable. | vx, steering_rate_limit(), reference_ds_min, model blending. | Use the built-in kinematic/dynamic blending and keep a permissive low-speed steering-rate limit. |

12. Recommended Development Workflow

Begin with the default VehicleParams and MPCParams and verify that the simulation runs without solver failures.

Tune the reference tracking weights before changing obstacle parameters. The vehicle should first follow an obstacle-free route reliably.

Add obstacles only after the basic path tracking is stable.

When changing speed, revisit steering-rate limits, lateral acceleration limits and obstacle look-ahead margins.

Use solver status, solve_time_ms, delta_seq and x_pred from MPCResult to diagnose numerical and control-quality problems.

Keep the QP structure in mind: very nonlinear or discontinuous modifications should be avoided unless they are linearised or represented through soft penalties.

13. Limitations and Assumptions

The longitudinal speed vx is not optimized by this module; it is an external input. A complete vehicle controller should include a separate longitudinal speed planner/controller.

Obstacles are circular and static in the current implementation.

The lateral acceleration constraint uses the simplified vx*r expression.

The model uses Euler discretisation and numerical Jacobian linearisation, which are simple and robust but not the most accurate possible methods.

The planner is mainly a lateral/path-tracking MPC. It does not include full road-lane geometry, traffic rules, moving obstacles or actuator dynamics beyond steering angle and steering-rate constraints.

15. Key Function Summary

| Function | Purpose |
| --- | --- |
| ReferencePath.from_xy() | Creates a reference path and computes heading values from X-Y coordinates. |
| extract_reference_window() | Extracts the local MPC horizon from the full reference path using arclength. |
| build_obstacle_aware_reference() | Shifts the local reference around obstacles and creates corridor constraints. |
| initial_guess() | Creates or warm-starts the predicted state and steering sequence. |
| build_qp() | Builds the sparse QP matrices for OSQP. |
| solve_mpc() | Runs the SQP/QP solution loop and returns MPCResult. |
| plan() | Main user-facing function for one MPC planning step. |
| step_vehicle() | Propagates the vehicle state with the internal discrete model. |
| steering_rate_limit() | Returns the speed-dependent steering change limit per sample. |